# Terminology and Concepts

Prescriptive framing: guidelines for building conventional light-frame wood structures without the need for engineering analysis of each component.

The two common types of roof construction:

Conventionally Framed

Truss System

Truss Systems use pre-fabricated trusses placed at prescribed spacing. Conventionally Framed Roofs, also known as stick-framed roofs, consists of repetitive rafters and ceiling joists also placed at prescribed spacing.

(Some definitions referenced from the AWC Wood Frame Construction Manual 2018)

Hip Rafter: A beam spanning from the ridge to the outside roof corner that supports the jack rafters, forming a convex roofline.

Jack Rafters: A rafter that spans from the hip or valley beam to a wall plate or ridge, respectively.

Valley Rafter: A beam spanning from the ridge to an inside roof corner, that supports the jack rafters, forming a concave roofline.

Ridge Beam: A structural member used at the ridge of a roof to support the ends of roof rafters and transfer roof loads to supports.

Ridge Board: A non-structural member used at the ridge of a roof to provide a nailing surface for the ends of roof rafters.

Continuous Tie

The joist to rafter and joist to joist connection creates a continuous tie along the joist span. This system withstands against rafter thrust that can happen when environmental or live loads are applied, such as people or snow.

Continuous Load Path

The interconnection of all framing elements of the lateral and vertical force resisting systems, which transfers vertical and lateral forces from their point of application to the foundation.

Rafter/Joist Connection

Requirements in the IRC necessitate increased nails to fasten ceiling joists to rafters, and ceiling joists to joist lap connections in order to resist rafter thrust. The connection between the rafter and joist is prescribed in Table R802.5.1(9).

## Roof Tie-Down

In the 2015 IRC, this connection has 50% more capacity compared to the 2009 IRC. This increase is because the fastening requirements have changed from two (2) toenails to three (3) toenails.  Where the uplift exceeds the capacity of the three nails, a roof to wall connector is required.

## For more information

2015 IRC: R802
Design Criteria

Selected Code Requirements from Chapter 8, 2015 IRC

- R802.2 Design and Construction The framing details required in Section R802 apply to roofs having a minimum slope of three units vertical in 12 units horizontal (25-percent slope) or greater. Roof-ceilings shall be designed and constructed in accordance with the provisions of this chapter and Figures R606.11(1), R606.11(2) and R606.11(3) or in accordance with AWC NDS. Components of roof-ceilings shall be fastened in accordance with Table R602.3(1).

- R802.3 Framing Details Rafters shall be framed not more than 1½-inches offset from each other to ridge board or directly opposite from each other with a gusset plate as a tie. Ridge board shall be not less than 1-inch nominal thickness and not less in depth than the cut end of the rafter.

At valleys and hips there shall be a valley or hip rafter not less than 2-inch nominal thickness and not less in depth than the cut end of the rafter.

Hip and valley rafters shall be supported at the ridge by a brace to a bearing partition or be designed to carry and distribute the specific load at that point.

Where the roof pitch is less than three units vertical, 12 units horizontal (25-percent slope), structural members that support rafters and ceiling joists, such as ridge beams, hips and valleys, shall be designed

as beams.

- R802.3.1 Ceiling joist and rafter connections Ceiling joists and rafters shall be nailed to each other in accordance with Table R802.5.1(9), and the rafter shall be nailed to the top wall plate in accordance with

Table R602.3(1).

Ceiling joists shall be continuous or securely joined in accordance with Table R802.5.1(9) where they meet over interior partitions and are nailed to adjacent rafters to provide a continuous tie across the building when such joists are parallel to the rafters.

Where ceiling joists are not connected to the rafters at the top wall plate, joists connected higher in the attic shall be installed as rafter ties, or rafter ties shall be installed to provide a continuous tie. Rafter ties will not be less than 2 inches by 4 inches, installed in accordance with the connection requirements in Table R802.5.1(9), or equivalent capacities shall be provided.

Where ceiling joists or rafter ties are not provided, the ridge formed by these rafters shall be supported by a wall or girder designed in accordance with accepted engineering practice.

Collar Ties or ridge straps to resist wind uplift shall be connected in the upper thirds of the attic space in accordance with Table R602.3.1. Collar ties shall be not less than 1 inch by 4 inches, spaced not more than 4 feet oc.

R802.3.2 Ceiling joists lapped Ends of ceiling joists shall be lapped not less than 3 inches (76mm) or butted over bearing partitions or beams and toenailed to the bearing member. Where ceiling joists are used to provide resistance to rafter thrust, lapped joists shall be tied together in accordance with R802.5.1(9) and butted joists shall be tied together in a manner to resist such thrust. Joists that do not resist thrust shall be permitted to be nailed in accordance with Table R602.3.1.

- R802.4 Allowable ceiling joist spans: Check spans for ceiling joists. Table R802.4(1) for uninhabitable attics without storage (Live Load = 10 psf) Table R802.4(2) for uninhabitable attics with limited storage (Live Load = 20 psf) For other grades and species and for other loading conditions, refer to the AF&PA Span Tables for Joists and Rafters.

- R802.5 Allowable rafter spans: Check Table R802.5.1(1) through R802.5.1(8) for rafter spans.

- Roof LL=20 psf, ceiling not attached to rafters

- Roof LL=20 psf, ceiling attached to rafters

- Ground snow load=30 psf, ceiling not attached to rafters

- Ground snow load=50 psf, ceiling not attached to rafters

- Ground snow load=30 psf, ceiling attached to rafters

- Ground snow load=50 psf, ceiling attached to rafters

- Ground snow load=70 psf, ceiling not attached to rafters

- Ground snow load=70 psf, ceiling attached to rafters

- R802.5.1 Purlins: Installation of purlins to reduce the span of rafters is permitted as shown in Figure R802.5.1. Purlins shall be sized no less than the required size of the rafters that they support.

Purlins shall be continuous and shall be supported by 2-inch by 4-inch braces installed to bearing walls at a slope not less than 45 degrees from the horizontal. The braces shall be spaced not more than 4 feet on center and the unbraced length of braces shall not exceed 8 feet.

- R802.6 Bearings: The ends of each rafter or ceiling joist shall have not less than 1-1/2 inches of bearing on wood or metal and not less than 3 inches on masonry or concrete.

The bearing on masonry or concrete shall be direct, or a sill plate of 2-inch minimum nominal thickness shall be provided under the rafter or ceiling joist. The sill plate shall provide a minimum nominal bearing area of 48 square inches.

- R802.7.1 Cutting and Notching of Sawn Lumber: Notches in solid lumber joists, rafters, blocking and beams shall not exceed one-sixth of the depth of the member, shall not be longer than one-third of the depth of the member and shall not be located in the middle one-third of the span. Notches at the ends of the member shall not exceed one-fourth the depth of the member.

The tension side of members 4 inches or greater in nominal thickness shall not be notched except at the ends of the members. The diameter of the holes bored or cut into members shall not exceed one-third the depth of the member. Holes shall not be closer than 2 inches to the top or bottom of the member, or to any other hole located in the member. Where the member is also notched, the hole shall not be closer than 2 inches to

the notch.

- R802.8 Lateral support: Roof framing members and ceiling joists having a depth-to-thickness ratio exceeding 5 to 1 based on nominal dimensions shall be provided with lateral support at points of bearing to prevent rotation. For roof rafters with ceiling joists attached per Table R602.3(1), the depth-thickness ratio for the total assembly shall be determined using the combined thickness of the rafter plus the attached ceiling joist. Exception: Roof trusses shall be braced in accordance with Section R802.10.3.

- R802.8.1 Bridging:  Rafters and ceiling joists having a depth to-thickness ratio exceeding 6 to 1 based on nominal dimensions shall be supported laterally by solid blocking, diagonal bridging (wood or metal) or a continuous 1-inch by 3-inch wood strip nailed across the rafters or ceiling joists at intervals not exceeding

eight feet.

- R802.9 Framing of openings: When the header joist span exceeds 4 feet, the trimmer joists and the header joist shall be doubled and of sufficient cross section to support the ceiling joists or rafter framing into the header. Approved hangers shall be used for the header joist to trimmer joist connections when the header joist span exceeds 6 feet.

- R802.11 Roof Tie-Down Roof assemblies shall have uplift resistance in accordance with Sections R802.11.1.1 and R802.11.1.2.

Where the uplift force does not exceed 200 pounds, rafters and trusses spaced not more than 24 inches on center shall be permitted to be attached to their supporting wall assemblies in accordance with Table R602.3(1).

Where the basic wind speed does not exceed 115 mph, the wind exposure category is B, the roof pitch is 5:12 or greater, and the roof span is 32 feet or less, rafters and trusses spaced not more than 24 inches on center shall be permitted to be attached to their supporting wall assemblies in accordance with Table R602.3(1).

- R802.11.1.2 Rafter Uplift Resistance Individual rafters shall be attached to supporting wall assemblies by connections capable of resisting uplift forces as determined by Table R802.11 or as determined by accepted engineering practice. Connections for beams used in a roof system shall be designed in accordance with accepted engineering practice.

Notes