Code Requirements for Conventionally Framed Roofs with

2015 IRC

Knowledge Assessment

| Question 1 (Slide 10) | Valley Rafters are structural members that need to be deeper than common rafters are because they are load bearing and need to provide full bearing for the rafter ends. - True - False |
| --- | --- |

| Question 2 (Slide 16) | - Roof systems are made with repetitive rafters and joists and should not be offset more than: - 24 inches - 18 inches - 5 inches - 1 ½ inches |
| --- | --- |

| Question 3 (Slide 34) | - Rafter ties are in_____________ when they are installed between rafters. - Compression - Tension - Reduction |
| --- | --- |

| Question 4 (Slide 47) | - The end of each rafter or ceiling joist shall have not less than 3 inches of bearing on wood or metal. - True - False |
| --- | --- |

| Question 5 (Slide 35) | - A joist located in the lower third of the attic is called a _________. - collar tie - rafter tie - hip rafter |
| --- | --- |

| Question 6 (Slide 39) | - A collar tie is a joist that is placed anywhere in the attic area. - True - False |
| --- | --- |

| Question 7 (Slide 56) | - When uplift is greater than ________, a design connection must be provided. - 24 inches on center - 115 mph - 150 lbs - 200 lbs |
| --- | --- |