Utilizing Mass Timber Metal Connectors Effectively

Knowledge Assessment

Note: For ICC members to receive ICC credits, they do not have to take this test. It is only required for general IACET CEUs that are self-reported to state agencies. Participants will receive an email with a link to take the test online. Participant will answer 7 of the 10 questions and must get 70% to pass the test.

- In which categories of buildings can CLT be mostly left exposed? (slide 26)

- Type IV-B to any height depending on occupancy

- All categories up to 8 or 9 stories depending on occupancy

- Type IV-C up to 8 or 9 stories depending on occupancy

- Type IV-C to any height depending on occupancy.

- Which of the following statements does NOT accurately describe mass timber buildings?  (slides 10,11.12,15)

A.  Mass timber buildings sequester carbon and mostly use timber from sustainably managed forests.

B.  Mass timber buildings reduce on-site construction time by approximately 40% and framing labor by up to 25%.

C.  Mass timber buildings can be built to at least 18 stories.

D.  Mass timber construction can exceed the strength and fire performance of other construction materials.

- Complete this statement. Hinge connectors can….. (slide  45)

- Transfer loads between two aligned beams

- Support vertical loads

- Resist horizontal loads

- Support joists on beams or walls

- 1 and 2 only

- 2 and 4 only

- 1, 2, and 3 only

- 3 and 4 only

- What are angle brackets used for? (slides 37 and 43)

- To connect a beam to a column

- To connect a column to a CLT wall panel

- To transfer vertical loads from the CLT panel to the beam.

- To connect CLT wall panels to CLT floors or concrete slabs.

- Which mass timber building classification permits the tallest buildings? (slide 28)

- Type IV-A

- Type IV-C

- Type IV-B

D    They all permit the same height.

- Identify the statement that most accurately describes column and plate design and construction.  (slides 53, 54, and 56)

- Column and plate construction utilizes plates resting on beams that are connected to columns.

- Column and plate construction provides the greatest flexibility in terms of column spacing.

- Column and plate design must size the CLT panels to provide sufficient resistance in the weaker span and at-point supports

- Column and plate construction is restricted to buildings of 8 or 9 stories.

- Which connection class mainly uses wood to protect the steel connector? ( slide 33)

- Class 3

- All classes do this

- Class 1.

- Class 2

- For which types of joints are connectors (screws) inserted at a 30° t0 40° angle? (slide 41)

- Half Lap Joint

- Butt Joint

- Tube Connectors

- Spline Joint

- Which of the following Mass Timber Types do NOT require glue for manufacturing?  (slides 17-20)

- NLT

- MLT

- CLT

- GLT

- What role does an anchor tie-down system perform? (slide 44)

- It ties down beams to columns to prevent uplift.

- It secures mid-rise, wood-framed buildings against over-turning forces.

- It anchors the mass timber columns to the concrete foundations.

- It attaches CLT floors to walls.