Diaphragm Design with 2015 SDPWS

Knowledge Assessment

| Question 1 (Slide 18) | - Match the term with the correct definition: - Chords :: Carry the axial force created by the moment (like flanges of a steel wide-flange beam) and are perpendicular to lateral load. - Collectors (Drag Struts) :: Boundary members that are parallel to the lateral load. They are designed to transmit horizontal diaphragm reactions to the shear walls. - Sheathing :: A layer of boards or other wood or fiber materials applied to the outer studs, joists, and rafters of a building to strengthen the structure. |
| --- | --- |

| Question 2 (Slide 19) | - Diaphragm shear capacity depends on: - Sheathing thickness and orientation - Fastener size and pattern - Framing lumber type and dimensions - All of the above |
| --- | --- |

| Question 3 (Slide 26) | - Tabulated nominal unit shear capacities for high load blocked wood structural panel diaphragms are provided in: - SDPWS Section 3.6.1 - IBC Section 2305.1.3.1 - SDPWS Table 4.2B - ASCE 7 Table 6-2 |
| --- | --- |

| Question 4 (Slides 29) | - What is the maximum aspect ratio for a blocked wood structural panel diaphragm? - 2:1 - 3:1 - 4:1 - No limit |
| --- | --- |

| Question 5 (Slide 32) | - Which of the following is not a component of Equation 4.2-1 for diaphragm deflection: - Tension and Compression - Chord Deformation - Panel Deformation and Nail Slip - Chord Splice Slip |
| --- | --- |

| Question 6 (Slide 39) | - True or false: A diaphragm may be idealized as rigid when the maximum in-plane deflection of the diaphragm is less than or equal to two times the average deflection of adjoining vertical elements. - True - False |
| --- | --- |

| Question 7 (Slide 40) | - The enveloped procedure for predicting the distribution of horizontal diaphragm forces uses the __________ of each scenario per wall line from flexible and rigid analyses. - Best case - Worst case - Average value (mean) - None of the above |
| --- | --- |

| Question 8 (Slides 41, 42) | - Choose whether the statement relates to either flexible or rigid diaphragms: - Distributes force to shear walls based on tributary area :: Flexible / Rigid - Force distributed to shear walls based on stiffness :: Flexible / Rigid - Easy to calculate :: Flexible / Rigid - Reduces forces on high-aspect ratio shear walls :: Flexible / Rigid |
| --- | --- |

| Question 9 (Slide 46) | - The ___________ in a diaphragm is used in conjunction with Tables 4.2A, 4.2B and 4.2C to find a viable sheathing and nailing combination. - Unit shear demand - Maximum deflection - Maximum shear demand - Maximum moment |
| --- | --- |

| Question 10 (Slide 48) | - Where is the maximum moment found on the deflected shape of a diaphragm? - Intersections of the chords and collectors - Distributed along the collectors - Connection points on the chords - Center of the chords |
| --- | --- |

| Question 11 (Slide 58) | - True or false: When a roof system without blocking is experiencing lateral forces, the roof sheathing will attempt to cause rotation and/or lateral displacement of the rafters/trusses. - True - False |
| --- | --- |

| Question 12 (Slide 61) | - Which of the following is a function of roof blocking at the plate line: - Provides a framing member for code-required nailing into diaphragm boundary element - Provides rotation and lateral displacement restraint of the rafter / truss - Transfers shear load from the diaphragm to the wall top plates (collectors) - All of the above |
| --- | --- |