SPDWS Shear Wall Design with the 2015 SDPWS

Knowledge Assessment

| Question 1 (Slide 10) | - True or false: When shear walls are designed properly, they are capable of transferring forces to the next element in the load path below them, such as other shear walls, floors, walls, slabs or footings. - True - False |
| --- | --- |

| Question 2 (Slides 20, 21) | - Tabulated nominal unit shear capacities for wood-based panels are provided in: - SDPWS Section 3.6.1 - IBC Section 2305.1.3.1 - SDPWS Table 4.3A - ASCE 7 Table 6-2 |
| --- | --- |

| Question 3 (Slide 25) | - SDPWS Section 4.3.1 defines permissible deflection as the deflection that: - Does not exceed 2” of inter-story drift - Permits the shear wall and attached elements to maintain their structural integrity and continue to support their prescribed loads - Allows the continuous load path to transfer loads to the other elements of the lateral-force-resisting system - All of the above |
| --- | --- |

| Question 4 (Slides 48) | - When is it most beneficial to use the new Equal Deflection Calculation (1.25-0.125h/bs)? - When you have a mixture of both low and high aspect ratio shear walls in a wall line. - When you have all low aspect ratio shear walls in a wall line. - When you have all high aspect ratio shear walls in a wall line. - When you are only analyzing a single shear wall in a wall line. |
| --- | --- |

| Question 5 (Slide 30) | - SDPWS Section 4.3.6.4.3 requires foundation anchor bolts to have a steel plate washer under each nut not less than 0.229”x3”x3” in size. Washers are necessary so that the __________ do not experience a __________. - Sill plates :: brittle failure - Sill plates :: ductile failure - Nails :: brittle failure - Nails :: ductile failure |
| --- | --- |

| Question 6 (Slide 39) | - True or false: When determining reduced capacities for shear walls with high aspect ratios, the 2bs/h factor must be applied simultaneously with the 1.25-0.125h/bs factor. - True - False |
| --- | --- |

| Question 7 (Slide 34) | - True or false: In the 2015 SDPWS, only seismic governed designs will need to consider adjustment factors for high aspect ratio shear walls. - True - False |
| --- | --- |

| Question 8 (Slide 34) | - The maximum permitted aspect ratio for a shear wall: - Varies based on the shear wall’s sheathing material. - Is not addressed in the 2015 SDPWS. - Is independent of the shear wall’s sheathing material. - Is always 1:1. |
| --- | --- |

| Question 9 (Slide 79) | - To find the Shear Capacity Adjustment Factor (Co) for a perforated shear wall, SDPWS Table 4.3.3.5 requires the designer to know the percent of full-height sheathing and the __________. - Maximum induced unit shear - Maximum opening height - Uniform uplift force - Tension and compression |
| --- | --- |

| Question 10 (Slide 104) | - True or false: When designing force-transfer shear walls, the definition of a wall pier changes depending on the height of the opening adjacent to it. - True - False |
| --- | --- |

| Question 11 (Slide 90) | - Out of the three common rational approaches for designing a force-transfer shear wall, which technique provided the most reasonable agreement with strap forces measured from full-scale testing? - Drag strut technique - Cantilever technique - Deikmann technique - None of the above |
| --- | --- |

| Question 12 (Slide 109) | - Which of the following is not true about pre-fabricated shear walls: - Usually the most economical solution - Allow for more openings in a structure - Help streamline quality, consistency and performance - Reduce design and labor time |
| --- | --- |