Wood Framed Seismic Design

Design Example Information

| Purpose | The purpose of including a design example is to help with the understanding and interpretation of the learning objectives. By applying the course material to a real-life scenario, we hope to improve your confidence when it comes to implementing these lessons in the field. By the end of Part 3 of Seismic Design for Wood Framed Construction, we will have designed the end-walls along gridlines 1 and 5 for seismic structural forces using the 2012 IBC allowable stress seismic provisions based on ASCE 7-10. In Part 1, we’ll put our design example to use by determining the seismic design parameters and finding the seismic design category. Slides that contain calculations or information regarding our design example will have indicating text shown on the bottom-right-hand corner. |
| --- | --- |

| Structure Information | - 3-story multi-family structure (Risk Category II) - Located in Sacramento, CA (38.634, -121.293) - Stiff Soil (Site Class D) - Entire building uses wood framing (studs and plate members are DF No. 1 or No. 2) - Roof trusses span the entire width - Conditions meet the parameters described in ASCE 7-10 Section 12.3.1, therefore a Flexible Diaphragm assumption is allowed |
| --- | --- |

| Building Plan | - Eight units on each level - Overall dimensions of 105’-4” x 65’-0” - Roof framing spans in the N-S direction - Symmetrical in both directions |
| --- | --- |

| Building Site Elevation | - Building repeats the floor plan at each level - Illustration shows the side of the high pitched gable roof |
| --- | --- |

| Building End Elevation | - Gable Roof has 6:12 pitch - All floors have an 8’ plate height with floor depths of 16” - Median roof height equals 35’-4” - Ridge height equals approximately 44’ |
| --- | --- |

| Seismic Design Parameters | SS | _______________ | S1 | _______________ |
| --- | --- | --- | --- | --- |
| (1/2) | Site Class | _______________ | Fa | _______________ |
|  | Fv | _______________ | SMS | _______________ |
|  | SM1 | _______________ | SDS | _______________ |
|  | SD1 | _______________ | Risk Category | _______________ |
|  | Seismic Design Category | _______________ |  |  |

| Seismic Design Parameters | Effective Seismic Weight |  | Response Modification Factor |  |
| --- | --- | --- | --- | --- |
| (2/2) | W | _______________ | R | _______________ |
|  | Wr | _______________ |  |  |
|  | W3 | _______________ |  |  |
|  | W2 | _______________ |  |  |
|  | Importance Factor |  | Seismic Design Coeffficient |  |
|  | Ie | _______________ | CS | _______________ |
|  | Fundamental Period |  | Long-Period Transition Period |  |
|  | Ta | _______________ | TL | _______________ |
|  | Seismic Shear |  | Lateral Seismic Force |  |
|  | v | _______________ | ∑Fx | _______________ |
|  | vasd_roof | _______________ | Fx_roof | _______________ |
|  | vasd_3rd | _______________ | Fx_3rd | _______________ |
|  | vasd_2nd | _______________ | Fx_2nd | _______________ |
|  | Redundancy Factor |  | Diaphragm Design Forces |  |
|  | ρ | _______________ | Fpx_roof | _______________ |
|  |  |  | Fpx_3rd | _______________ |
|  |  |  | Fpx_2nd | _______________ |
|  | Diaphragm Nominal Capacity |  | Chord Force |  |
|  | vs | _______________ | T | _______________ |

Notes / Solution