Wood Framed Seismic Design

Knowledge Assessment

| Question 1 (Slide 27) | Per IBC 2012 Section 1613.5.2 (Exception): If soil properties are not known in sufficient detail, then Site Class ____ shall be used unless the building official or geotechnical data determines a higher Site Class soil is likely present at the site. - Site Class A - Site Class B - Site Class C - Site Class D |
| --- | --- |

| Question 2 (Slide 34) | Which organization’s website should you visit to find the seismic design parameters for the site of your project? - CNN - USGS - Farmer’s Almanac - Simpson Strong-Tie |
| --- | --- |

| Question 3 (Slide 33) | The Seismic Design Category of a structure is based on which two values? - Seismic Use Group and Importance Factor (Ie) - Soil Site Class and Site Coefficient (Fa & FV) - Risk Category and Design Spectral Response Acceleration Parameters (SDS & SD1) - Risk Category and Importance Factor (Ie) |
| --- | --- |

| Question 4 (Slide 23) | What aspects of design can be revealed by knowing the Seismic Design Category? - Permissible structural system - Limitations on height and irregularity - Types of lateral force analysis that may be used - Special inspection requirements - All of the above |
| --- | --- |

| Question 5 (Slide 40) | In which Seismic Design Categories do the design forces need to be increased by 25% for connections of diaphragms to vertical elements and to collectors, and for connections of collectors to vertical elements per ASCE 7-10 Section 12.3.3.4? - Seismic Design Categories A, B, and C - Seismic Design Categories E and F only - Seismic Design Categories A and B only - Seismic Design Categories D, E, and F - None of the above |
| --- | --- |

| Question 6 (Slide 50) | The Loma Prieta earthquake of 1989 and the Northridge earthquake of 1994 were the catalysts for exposing the nation to the dangers of which vertical irregularity? - Stiffness irregularity (soft story) - Weight (mass) irregularity - In-plane discontinuity with vertical lateral force-resisting element irregularity - Vertical geometric irregularity |
| --- | --- |

| Question 7 (Slide 63) | Which two parameters comprise the Seismic Base Shear? - Response Modification Factor and Effective Seismic Weight - Effective Seismic Weight and Seismic Response Coefficient - Seismic Response Coefficient and Importance Factor - Importance Factor and Response Modification Factor |
| --- | --- |

| Question 8 (Slide 73) | What is the Response Modification Factor, R, for light framed structures with wood structural shear panels used for the basic seismic force-resisting system? - R = 4.5 - R = 6.0 - R = 6.5 - R = 3.0 - None of the above |
| --- | --- |

| Question 9 (Slide 87) | The Vertical Distribution Factor, Cvx, is dependent upon the structure weight (at each level), the height of the structure (at each level), and the structure’s period. - True - False |
| --- | --- |

| Question 10 (Slide 98) | What is the maximum horizontal diaphragm dimension ratio allowed if the diaphragm is constructed of wood structural panels and nailed at all edges (with no unblocked edges)? - 6:1 - 5:1 - 4:1 - 3:1 |
| --- | --- |

| Question 11 (Slide 104) | Blocking between roof trusses (or roof rafters) allows for edge nailing of the roof diaphragm and gives a direct load path for shear transfer. Blocking will also help the designer prevent which of the following structural concerns: - Rotation of the truss/rafter and lateral displacement of the truss/rafter - Rotation of the truss/rafter and uplift of the truss/rafter - Uplift of the truss/rafter and lateral displacement of the truss/rafter - Uplift of the truss/rafter and obstructing eve ventilation - None of the above |
| --- | --- |

| Question 12 (Slide 107) | Which of the following does not contribute to diaphragm deflection? - Panel shear and fastener slip deflection - Chord splice slip deflection - Perpendicular to grain compression - Bending deflection - None of the above |
| --- | --- |

| Question 13 (Slide 151) | According to the SDPWS Section 2305.3.11 for seismic design, which size bearing plate is required with foundation anchor bolts in order to prevent sill plate splitting? - 2” x 2” x 3/16” - 3” x 3” x 0.229” - 3” x 3” x 3/16” - 2” x 2” x 0.229” - None of the above |
| --- | --- |

| Question 14 (Slide 124) | Which of the three methods described in ASCE 7-10 for analyzing wood shear walls requires special detailing around each opening that is within a shear wall? - Segmented Shear Wall Method - Perforated Shear Wall Method - Force Transfer Method - Torsional Equilibrium Method |
| --- | --- |

| Question 15 (Slide 123) | How would a designer install holdowns on a segmented wood shear wall that is adjacent to an opening? - One holdown per shear wall segment; installed in the center of the segment - Two holdowns per shear wall segment; one installed at each end of the segment - Zero holdowns required for segmented shear walls - Three holdowns per shear wall segment; one installed at each end of the segment and one installed in the center of the segment |
| --- | --- |

| Question 16 (Slide 127) | Perforated shear walls have a maximum allowable height of: - 10 feet - 15 feet - 20 feet - 30 feet |
| --- | --- |

| Question 17 (Slide 132) | Toenails cannot transfer more than __________ of shear from floor to floor in Seismic Design Category D, E, or F. - 100 plf - 120 plf - 150 plf - 180 plf - 200 plf |
| --- | --- |

| Question 18 (Slide 156) | The allowable story drift of a structure is dependent on: - Risk Category and type of construction material used for the seismic force-resisting system - Importance Factor and Mapped Acceleration Parameters - Deflection Amplification Factor and Importance Factor - Overturning Moment and the Effective Seismic Weight - None of the above |
| --- | --- |