Deck Inspection for New and Existing Construction

Knowledge Assessment

| Question 1 (Slide 18) | - If any component of a deck is deemed unsafe per the code, it must be repaired or replaced regardless if the deck has passed a previous inspection. - True - False |
| --- | --- |

| Question 2 (Slide 21) | - The life expectancy of a deck ranges from: - 5 to 10 years - 10 to 20 years - 15 to 30 years |
| --- | --- |

| Question 3 (Slide 31) | - Which of the following should not be used to positively anchor the deck ledger to the rim board? - Bolts - Toenails - Lag screws - Proprietary structural wood screws |
| --- | --- |

| Question 4 (Slide 77) | - Coating types and weights for connectors with preservative-treated wood shall be in accordance with the connector manufacturer’s recommendation. In the absence of a manufacturer’s recommendation, a minimum of _______________, or equivalent, shall be used. - Standard G90 zinc coating - ASTM A 653, type G185 zinc-coated galvanized steel - Type 316L stainless steel - Powder coating |
| --- | --- |

| Question 5 (Slide 40) | - A guard is required on _______________ located more than 30 inches above the floor or grade below. - Porches and balconies - Ramps - Raised floor surfaces - All of the above |
| --- | --- |

| Question 6 (Slide 51) | - Deck footings shall be placed not less than __________ below the undisturbed ground surface. - 6 inches - 9 inches - 12 inches - 18 inches |
| --- | --- |

| Question 7 (Slide 59) | - The building codes place a limit on the size of openings, for the occupants’ safety in case of tripping or falling, and for the safety of small children. The codes require that a __________ sphere must not be able to pass between the openings of risers. - 4-inch - 5-inch - 6-inch - 7-inch |
| --- | --- |

| Question 8 (Slide 68) | - Nails, screws and spikes loaded in withdrawal from the end grain of the wood has been prohibited since the 1944 edition of the NDS. - True - False |
| --- | --- |

| Question 9 (Slide 36) | - The ledger must be installed over _______________ or directly into the rim joist or stud. - Stucco - Siding - Structural sheathing - All of the above |
| --- | --- |

| Question 10 (Slide 42) | - The guard rail system shall be able to resist _______________ applied in any direction at any point along the top. - 100 pounds - 200 pounds - 300 pounds - 400 pounds |
| --- | --- |

| Question 11 (Slide 61) | - Codes require that a _____ sphere must not be able to pass through the triangular openings of a notched stringer formed by the riser, tread and bottom rail of a guard. - 4-inch - 6-inch - 7-inch - 9-inch |
| --- | --- |

| Question 12 (Slide 69) | - Unless otherwise noted, bending steel in the field may cause fractures at the bend line. Fractured steel will not carry the load and must be replaced. - True - False |
| --- | --- |