DEMO #1: Drilling and Anchoring

9:15am-10:00am

45 minutes total. Spilt class into 2 groups; start one group off at each station, switch after 20 mins and repeat demo. (NOTE: Ideal class size is 20)

|  | Station 1 | Station 2 |
| --- | --- | --- |
| Round 1 | Drill holes & install THD | Install SET-XP, AT-XP, SB2 |
| Round 2 | Drill holes & install THD | Install SET-XP, AT-XP, SB2 |

Concrete: Each station should have a pad of concrete (pallet size) – use HALF one pad. Save the other half for DEMO #2: Concrete Repair to take place later in the day.

STATION 1:

Purpose:

- Showcase Bosch tools – cordless hammer, cordless drills, impact wrenches to set anchors

- Show THD can be driven easily and requires less torque, even larger diameter anchors

Materials/Demo:

- Drill holes 3/8” x 5” deep for installing 3/8” x 3” THD37300H

- Drill hole and install Titen HD rod hanger – 3/8” hole THD50234RH

- Drill larger diameter  ¾” x 6” hole to install THD75400H

STATION 2:

Purpose:

- Show application and features of SET-XP and AT-XP

Preparation:

- Before class, pre-drill holes

  - 5/8” x 4” holes – at least one for every attendee for adhesive installs

  - 3/8” x 3 ½” hole – cleaned out for Strong Bolt 2 installs

Materials/Demo:

SET-XP and AT-XP Installation

- SET-XP22

- EMN22I (nozzle)

- EDT22S (dispensing tool)

- SET-XP10

- AT-XP10

- CDT10S (dispensing tools)

- RFB#4X6

- ETB4 (brushes)

Strong Bolt 2 Installation

- STB2-37500 (anchor)

- Hammer

- Torque wrench and socket