DEMO #2: Concrete Repair

11:15am-12:15pm

1 hour total. Spilt class into 2 groups; start one group off at each station, switch after 30 mins and repeat demo. (NOTE: Ideal class size is 20)

|  | Station 1 | Station 2 |
| --- | --- | --- |
| Round 1 | Chipping, grinding | FX-928, FX-263 |
| Round 2 | Chipping, grinding | FX-928, FX-263 |

STATION 1:

Purpose:

- Showcase Bosch grinders, chippers, dust collection

Concrete: Same slab of concrete where holes were drilled and THD installed – use other half to demo surface prep using Bosch chipper and grinders.

STATION 2:

Purpose:

- Demo how quickly FX-928 sets

- Demo properties of FX-263

Concrete: Same slab of concrete where adhesives and SB2 were installed – use other half to demo FX-263.

Preparation:

- Display to create ahead of time: Concrete with rebar already primed, and bonding agent partially applied.

- Chip away half of the slab where epoxies will be installed so it is ready to demo patching

- Need box to pour concrete mix into

Materials/Demo:

- 3 bags of FX-928 – pour into empty box, smooth out with trowel

- 1 sample size FX-263 – patch up chipped away concrete

- (4) 5-gallon buckets

- (2) 1 quart measuring cups

- 2 margin trowels

- 1 rectangle trowel

- Box of nitrile gloves

- Water

- Box of rags

- Drop cloths or tarp

- 50 ft extension cord

- Mixer

- Chip brushes (to saturate/wet the concrete before using FX-263)